LOGICAL DEVICES,INC.

LOGICAL DEVICES EXE/COM FILE CONVERSION  UTILITIES

The  two files supplied on this disk can be used to convert  a

binary  file (.EXE or .COM ) to a standard Intel Hex format  used

to  program  an  EPROM.  This format is used on  any  of  Logical

Devices programmers.

.COM File Conversion

For conversion of a .COM file the HEXCON utility will be used.

To  start the program place the disk in an appropriate drive.  At

the  prompt type HEXCON then press return.  A menu  will  appear,

after  entering  all information a new Hex file will be  created.

This  new hex file can be used to download to the programmer  and

create the ROM version of the program.

Note: Entering CON for the output file name will send the Hex

file to the screen instead of a disk file.

.EXE File Conversion

For  conversion  of an .EXE file the EXE2HEX utility  will  be

used.  To  use  this utility type EXE2HEX FILENAME  SEGMENT  then

press return.

FILENAME = the file to be converted

Note: Do not place an extension on the filename as

a .EXE will be automatically appended.

SEGMENT = the base address the file is to be created

Example:  a  file  to be converted to hex  address

02000h the SEGMENT would be 2000

An Example of converting an EXE file to a Hex file.

To convert the file PROGRAM.EXE to a Hex file starting at  0E000h

type:

A>EXE2HEX PROGRAM E000

1

The  file PROGRAM.HEX will be created with a starting  address

of 0E000h.

Note:     Most programs when placed on EPROM need to be loaded at

location  0000h.  Even if the processor may assign  the

ROM code a higher address.  The EPROM will be addressed

at  this assigned location but the data needs to be  at

the  beginning  of  the ROM.  Therfore a  segment  base

address  of  0000h should be used on  both  conversions

unless  the  user specifically designs the data  to  be

placed at various locations other than 0000h.

2